

1) : MBS approves booth design is based on the overall structural concept.

2) : The exhibitor and appointed contractor have to ensure the structure integrity of their booth based on their method of construction with due diligence and in compliance with all laws.

3) : Maximum height allowed in the Expo Hall is 7mHt.

4) : Maximum height allowed in L3 & L4 ballroom is 4mHt. While  L5 Grand Ballroom is 6mHt. ( excluding public space )

5) : P.E (  Professional Engineer ) endorsement is required for any structure measured at 4m from ground.

6) : P.E  ( Professional Engineer ) endorsement must be submitted for any LED wall build up from floor with surface area more than 10m².

7) : MBS reserves the right to request for P.E endorsement of structure regardless of any design or height.

8) : Licensee/Organizer to file all Risk Assessment when conducting high risk works such as work at height (above 2mHt.) and lifting works under the current WSH act ( MOM ).

9) : Fire retardant certificate to be submitted for all fabric installation and has to be water permeable.

10) : Fire sprinkler ball to be installed at every 9sqm of covered ceiling.

11) : All raw space booth to install plywood underlay for construction on all carpeted space.

12) : All stand materials (plywood underlay/wooden platform) to have plastic underlay as protection on all carpeted space.

13) : Edge protection is required for all edges of platform and plywood underlay.

14) : Client to make necessary provision if edge of platform poses any forms of tripping hazard.

15) : Scaffold usage needs to produce (a) Work At Height licence (b) Scaffold Erector licence (c) Inspection tag

16) : Booth dismantling : Must be done is an organised manner, no collapsing method is allowed.